# Mechanical Design Strategy

- Use standardized mass-produced materials first

AIAM should build on top of existing mass-production supply chains.

Preferred base materials:

- aluminum extrusion profiles

- standard linear rails

- standard bearings

- standard lead screws/ball screws

- standard fasteners

- 3D-printed functional plastic parts

- simple 2D-cut metal plates

Design principle:

Use custom 3D CNC parts only when absolutely necessary.

The default structure should be:

Metal extrusion with 2D fabrication + 3D printed plastic + standard hardware.

This makes the machine easier to build, repair, copy, and upgrade.

- Replace 3D CNC parts with hybrid parts

Instead of machining one complex aluminum block, AIAM should divide the function into simpler pieces:

### Metal portion

Used for:

- load-bearing surfaces

- deformation resistance

- threaded inserts

- precision mounting surfaces

- wear surfaces and corners

- heat resistance

### Plastic / 3D-printed portion

Used for:

- shape complexity

- covers

- cable routing

- sensor holders

- quick-change brackets

- alignment guides

- lightweight structure

- fast iteration

### Bonded/screwed assembly

The final part can be assembled using:

- screws

- heat-set inserts

- dowel pins

- adhesive bonding

- metal reinforcement plates

- replaceable wear inserts

Design principle:

Do not machine complexity. Print complexity. Cut strength. Assemble function.

This is a very strong AIAM concept.

- Prefer simple 2D fabrication

Simple 2D fabrication means:

- 2D router cutting

- laser-cut plates

- waterjet-cut plates

- saw-cut aluminum profiles

- drilled hole patterns

- bent sheet metal

This reduces cost and improves repeatability.

Good AIAM mechanical design should look like:

flat plates + extrusion + printed adapters + modular actuators.

Avoid unnecessary organic or complex 3D-machined geometry.

- Design for upgrade and extension

Every AIAM machine should be easy to extend.

Mechanical rules:

- use repeated hole patterns

- use standard extrusion slots

- leave mounting zones for future sensors

- make actuator modules replaceable

- separate frame, actuator, sensor, and tool sections

- make covers removable

- avoid hidden fasteners when possible

- avoid permanent custom geometry unless needed

Design principle:

A machine is not finished. It is a platform for the next function.