Motion Control Strategy

- Eliminate pneumatic actuators when possible

AIAM should avoid pneumatic cylinders as a default choice.

Replace them with:

- stepper linear actuators

- BLDC actuators

- servo actuators

- lead screw modules

- belt-driven modules

- small electric grippers

- solenoid only for very simple low-force motion

Pneumatics can still be used only when there is a strong reason, such as:

- very high force

- dirty environment

- simple clamp with existing air supply

- customer requirement

But AIAM default should be:

Electronic actuator first. Pneumatic only by exception.

- Software-controlled motion reduces mechanical cost

Electronic actuators allow:

- speed control

- position control

- acceleration control

- force/current estimation

- soft landing

- repeatable motion profiles

- self-diagnosis

- flexible recipe change

This means the mechanical structure can be lighter because the actuator does not create unnecessary impact.

Design principle:

Use software to remove shock from the machine.

- Use PID and input shaping

For AIAM machines, motion should not simply be “move fast.”

Motion should be designed.

Use:

- PID position/speed control

- jerk-limited motion

- S-curve acceleration

- input shaping

- vibration reduction profiles

- current monitoring

- stall detection

- homing and calibration routines

Main goal:

Quiet motion, low vibration, long machine life.

This supports the use of non-metal or hybrid parts.